# High Level Architecture

For the DIY Smart Lock based on Arduino, the high-level architecture involves integrating RFID technology with an Arduino to create a secure access control system. Here's a detailed breakdown of the high-level architecture for the DIY Smart Lock Project:

### 1. Hardware Components:

### a. Arduino Board:

- Type: Arduino Uno (or compatible).

- Function: Central controller for processing RFID data, managing access, and controlling the lock mechanism.

### b. MFRC522 RFID Module:

- Function: Reads RFID card data and communicates with the Arduino.

- Connection: Connected to specific pins on the Arduino for data exchange.

### c. LEDs:

- Colors: Red, Green, and Blue.

- Function: Visual indicators for system status (access granted, access denied, admin mode).

### d. Resistors:

- Function: Protects components, like the LEDs, from excessive current.

### e. MOSFET:

- Function: Controls the solenoid lock, acting as a switch triggered by the Arduino.

### f. Solenoid Lock:

- Function: Physical lock mechanism controlled by the Arduino to secure/unlock access.

### g. Power Supply:

- Voltage: Typically 12V.

- Function: Powers the solenoid and Arduino.

### h. Breadboard and Wires:

- Function: Provides a platform for temporary circuit connections.

### 2. Interconnections:

- The MFRC522 module is connected to specific digital pins on the Arduino.

- LEDs are connected to digital output pins for visual feedback.

- The solenoid is controlled by a MOSFET, connected to a dedicated pin on the Arduino.

### 3. Software Components:

### a. Arduino IDE:

- Function: Used for programming the Arduino board.

### b. RFID Library (e.g., MFRC522 Library):

- Function: Enables communication with the MFRC522 RFID module.

### c. Access Control Sketch:

- Logic: Contains code for RFID card detection, authentication, access control, LED feedback, and lock control.

- Structure: Utilizes functions for different tasks (e.g., accessControl(), authenticateCard(), grantAccess()).

### 4. Operation Logic:

- The Arduino continuously checks for the presence of RFID cards using the MFRC522 module.

- Upon card detection, the system checks for the master card to enter admin mode or authenticates regular cards.

- Access is granted by activating the solenoid and displaying green LED feedback.

- Access denial triggers the red LED, followed by a delay and LED reset.

- Admin mode activates the blue LED, providing additional functionalities.